Mountain Cottage
ARC171

Download the Mountain Cottage drawings and print out. This has all needed dimensions on it top complete your drawing.

1. Using 3H lead and a sheet of 24” x 36” vellum drawing paper, draw the plan, front elevation and side elevation of the building. Be sure to cross lines at corners.

2. Go over the guidelines in H or F lead, showing at least five unique line weights:

a. Guidelines

b. Texture lines

c. Object lines

d. Profile lines

e. Ground lines

3. Outline shadows in plan, front and side elevations

4. Color using Prismacolor pencils on back of drawing. 
5. Show shades and shadows by Prismacolor pencils shading in plan, front and side elevations

6. Trace title block

7. Mount on foam core board

Follow Quality Assurance Rules at http://academics.triton.edu/faculty/fheitzman/QA.html 
